A

PROJECT REPORT ON

“ROOMATE AI”

SUBMITTED BY:

MEET AGRAWAL(2303466160046) CHAVDA ADITYASINH (230346160070) KRISHNA SHARMA(2303466160252)

GUIDED BY:

Prof. POONAM FALDU

DIPLOMA IN COMPUTER ENGINEERING

SUBMITTED TO:

PARUL POLYTECHNIC INSTITUTE

POST – LIMDA, WAGHODIA – 391760

PARUL POLYTECHNIC INSTITUTE

COMPUTER ENGINEERING DEPARTMENT

LIMDA, WAGHODIA, VADODARA

# C E R T I F I C A T E

This is to certify that CHAVDA ADITYASINH, MEET AGRAWAL & KRISHNA SHARMA students of Computer Engineering, Enrolment No: 2303466160070 , 2303466160046 & 2303466160252 have satisfactorily completed their project work as a part of course curriculum in Diploma in (5th Semester) having a project title “ROOMATE AI”.

#### Signature of Guide

ASST. PROF. POONAM

FALDU

#### Head of Department

ASST. PROF. POONAM

FALDU

PARUL POLYTECHNIC INSTITUTE

COMPUTER ENGINEERING DEPARTMENT

LIMDA, WAGHODIA, VADODARA

# C E R T I F I C A T E

This is to certify that CHAVDA ADITYASINH, MEET AGRAWAL, KRISHNA SHARMA students of Computer Engineering, Enrolment No: 2303466160070, 2303466160046, 2303466160252 have satisfactorily completed their project work as a part of course curriculum in Diploma in (5th Semester) having a project title ROOMATE AI.

DATE:

INTERNAL JURY:							EXTERNAL JURY:

ACKNOWLEDGEMENT

###### We take this opportunity to express our profound sense of gratitude and respect to all

those who helped us throughout the duration of this project.

Firstly, we are extremely grateful to Parul Polytechnic Institute, for providing us the

###### excellent working environment to undergo our project.

We devote our success in this effort to our project guide Prof. Poonam Faldu for giving us the opportunity to undertake the project and providing crucial feedbacks that influenced us and provided opportunity to undertake the project work in the esteemed concern.

We are also deeply thankful to Prof. Poonam Faldu, Head of Computer Engineering Department, whose useful suggestions, gentle soothing attitude and right directions helped us a lot to learn in this project and also for her constant encouragement and support throughout the project.

Last, but not the least, we would like to extend our profound thanks to all our esteemed colleagues and friends at college level who helped us in the specific areas of this project.

Yours Sincerely,

Chavda Adityasinh (2303466160070)

Meet Agrawal (2303466160046)

Krishna Sharma (2303466160252)

ABSTRACT

RooMate AI is an intelligent and adaptive smart room assistant designed to provide secure, convenient, and personalized room automation. The system integrates computer vision, IoT, and machine learning techniques to create a unified platform for smart living. It features a gesture- controlled interface using OpenCV and MediaPipe, allowing users to manage appliances such as lights, fans, and media systems without physical switches.

The hardware backbone is powered by ESP32 microcontrollers with relay modules, enabling reliable wireless IoT control of connected devices. Beyond manual control, RooMate AI incorporates a rule- based behavior learning engine that observes user habits, predicts routine activities, and automates repetitive tasks like switching off unused devices or controlling home appliances. A real-time dashboard further enhances usability by displaying device status, usage logs, and system alerts, ensuring transparency and remote accessibility.

To enhance safety and flexibility, the system also integrates a gas leakage detection module that provides instant alerts during hazardous situations, along with a web-based application that allows users to manually control appliances, access camera-based gesture control through a smartphone, and monitor system performance remotely.

The system is designed to be low-cost, scalable, and energy-efficient, making it suitable for deployment in homes, hostels, and office environments. By combining gesture recognition, behavior learning, IoT integration, robust security, and safety monitoring, RooMate AI represents a step toward creating intelligent environments that adapt to human needs, reduce energy wastage, and improve overall quality of life.

INDEX

| Sr.no |  | Description | Page No |
| --- | --- | --- | --- |
| Chapter 1 |  | Introduction | 1 |
|  | 1.1 | Project Summary | 2 |
|  | 1.2 | Purpose | 3 |
|  | 1.3 | Scope | 4 |
|  | 1.4 | Benefits | 5 |
| Chapter 2 |  | System requirement study | 6 |
|  | 2.1 | Hardware Requirements | 7 |
|  | 2.2 | Software Requirements | 8 |
|  | 2.3 | Functional Requirements | 9 |
| Chapter 3 |  | Diagram | 10 |
|  | 3.1 | Circuit Diagram | 11-12 |
|  | 3.2 | System Design | 13 |
| Chapter 4 |  | Data Dictionary | 14 |
|  | 4.1 | Data Source | 15-16 |
|  | 4.2 | Data types | 17 |
|  | 4.3 | Data & Organization | 18-19 |
| Chapter 5 |  | Limitations & Future Enhancement | 20-22 |
| Chapter 6 |  | Conclusion | 23-24 |

Roomate AI

CHAPTER 1 :INTRODUCTION

- 1.1 Project Summary

- 1.2 Purpose

- 1.3 Scope

- 1.4 Benefits

1

Roomate AI

Chapter : 1 Introduction

  - Project Summary

RoomMate AI is a smart room automation system designed to provide intelligent, adaptive, and secure control of a room’s environment. It combines computer vision, IoT, and sensor technologies to deliver a user-friendly, efficient, and safe smart living experience. The key aspects of the project include:

Gesture-Controlled Appliance Management: Users can control lights, fans, and other appliances using hand gestures, captured via a smartphone camera and processed using OpenCV and MediaPipe.

    - Gas Leakage Detection: A gas sensor continuously monitors for hazardous gas levels, sending instant alerts through the system to prevent accidents.

    - IoT-Based Automation: ESP32 microcontrollers with relay modules enable wireless control of

connected devices, ensuring reliable and responsive automation.

    - Web Application Integration: The system includes a web-based dashboard allowing manual control of appliances, real-time monitoring, gesture control via camera, and alert notifications.

    - Behavior Learning Engine : RoomMate AI can learn user habits over time to automate repetitive tasks

###### like turning off unused devices.

    - Energy Efficiency: Automated control of devices reduces unnecessary power consumption, contributing to sustainable living.

    - Enhanced Security: By minimizing manual interaction with switches and incorporating real-time monitoring, the system reduces human error and potential hazards.

    - Modular and Scalable Design: The hardware and software are designed for easy expansion, allowing

integration of additional sensors, appliances, or AI features in the future.

    - User-Friendly Experience: Combining gesture-based control, web access, and real-time alerts ensures a seamless and convenient interface for users of all ages.

    - Deployment Versatility: Suitable for homes, hostels, offices, and small commercial spaces, making it adaptable to various environments.

    - Cost-Effective Solution: Uses affordable hardware while providing advanced automation and safety features, making smart living accessible.

Roomate AI

  - Purpose

The primary purpose of RoomMate AI is to develop an intelligent, adaptive, and user-friendly room automation system that enhances safety, convenience, and energy efficiency in modern living spaces. The system aims to combine gesture-based control, IoT automation, and real-time monitoring to offer a comprehensive solution for both daily convenience and emergency situations.

    - Hands-Free Control of Appliances: Enable users to operate lights, fans, and other electrical appliances without physical switches, using natural hand gestures captured via a smartphone camera. This reduces manual effort and makes the system accessible to individuals with mobility challenges.

    - Enhancing Safety: Integrate a gas sensor module to detect hazardous gas leaks and immediately trigger alerts. The system ensures timely preventive measures, reducing the risk of accidents and improving user safety.

    - Flexible Control Options: Provide a web application interface that allows users to manually control appliances, monitor device status, and access gesture control remotely, ensuring flexibility and convenience.

    - Automation and Behavior Adaptation: Incorporate a rule-based behavior learning engine that observes usage patterns and automates repetitive tasks such as switching off idle devices, adjusting fan speeds, or managing room lighting according to user preferences.

    - Real-Time Monitoring and Alerts: Maintain a dashboard that displays device status, usage logs, safety alerts, and system notifications to ensure users have complete awareness and control over their environment.

    - Affordable and Scalable Smart Solution: Design a cost-effective, modular system that can be deployed in homes, hostels, offices, or small commercial spaces, and easily upgraded with additional appliances, sensors, or features in the future.

    - Energy Efficiency and Sustainability: Optimize energy consumption by automating device operations based on usage patterns, reducing unnecessary power wastage, and promoting environmentally responsible living.

    - Modernizing Everyday Living: Enhance the quality of life by combining convenience, safety, adaptability, and intelligence in a single system, demonstrating how technology can create a smarter and more comfortable living environment.

Roomate AI

#### Scope

The scope of RoomMate AI encompasses the design, development, and deployment of a smart room automation system that combines gesture-based control, IoT hardware, safety monitoring, and software integration to improve convenience, security, and energy efficiency. The system is designed to cater to homes, hostels, offices, and other small-scale environments.

    - Gesture-Based Appliance Control: Using OpenCV and MediaPipe, the system allows users to control electrical appliances such as lights and fans through hand gestures captured by a smartphone camera, reducing the need for physical interaction.

###### IoT Integration: Employs ESP32 microcontrollers and relay modules for reliable wireless control of

connected devices, ensuring real-time and responsive automation.

    - Gas Leakage Detection: Incorporates a gas sensor module that monitors the room environment for hazardous gas leaks and triggers instant alerts, contributing to occupant safety.

    - Web Application Interface: Includes a user-friendly web-based dashboard that allows users to:

###### Manually control appliances

- Access real-time device status and activity logs

- View alerts from the gas sensor

- Control devices remotely and interact with gesture-based controls via mobile camera

    - Behavior Learning and Automation: Supports rule-based automation that can adapt to user habits over time, such as:

- Switching off unused devices automatically

- Adjusting fan speed or lighting based on user patterns

###### Reducing energy consumption and enhancing comfort

    - Real-Time Monitoring: Provides a comprehensive dashboard for monitoring device operation, safety alerts, and system performance, ensuring transparency and control for the user.

Roomate AI

#### Benefits

RoomMate AI provides numerous benefits across convenience, safety, energy efficiency, and adaptability:

    - Hands-Free Operation – Gesture-based control allows users to operate lights, fans, and other

appliances without physical contact, making it convenient and accessible.

    - Enhanced Safety – The gas leakage detection module ensures timely alerts in case of hazardous

###### situations, preventing accidents.

    - Flexible Control Options – The web application dashboard allows manual control, monitoring, and camera-based gesture control, offering remote access and convenience.

    - Energy Efficiency – Automation and habit-based controls help reduce unnecessary electricity usage, promoting sustainable living.

    - Automation and Habit Learning – The system can observe user patterns and automate repetitive

tasks like turning off idle devices or adjusting lighting/fan levels.

    - Cost-Effective – Uses affordable hardware to provide advanced smart room features, making it practical for wide adoption.

    - Scalability – Modular design allows integration of additional devices, sensors, or AI features in the

###### future.

    - Enhanced User Experience – Combines convenience, comfort, and security in a single system for a modern smart living environment.

    - Versatile Deployment – Suitable for homes, hostels, offices, and small commercial spaces.

    - Educational and Research Value – Serves as a learning platform for IoT, computer vision, and smart automation projects.

Roomate AI

## Chapter 2 System requirement study

      - 2.1 Hardware Requirement

      - 2.2 Software Requirements

      - 2.3 Functional Requirements

Roomate AI

Chapter 2 System Requirement Study

#### Hardware Requirements

The hardware forms the backbone of RoomMate AI, enabling gesture-based control, IoT automation, and safety monitoring. Each component is carefully selected to ensure reliable operation, cost-effectiveness, and scalability.

    - ESP32 Microcontroller : Acts as the central processing unit for the entire system.

    - Handles communication between the web application, relay modules, sensors, and gesture inputs.Supports Wi-Fi and Bluetooth, making it suitable for wireless IoT control.

    - Lightweight and low-power, allowing efficient energy usage.

    - Relay Modules : Serve as switching interfaces between the microcontroller and high-power appliances like lights and fans.Convert the low-voltage control signal from ESP32 into ON/OFF switching for electrical devices.Ensure safe operation without damaging sensitive electronics.

    - Gas Sensor (MQ-2 / MQ-5) : Monitors the room for hazardous gases such as LPG, methane, or smoke.Triggers instant alerts via the web application when gas concentration crosses safe levels.Ensures user safety and prevents potential accidents in the room.

    - Smartphone / Camera : Provides the visual input for gesture recognition.Captures real-time hand movements for controlling lights and fans via OpenCV and MediaPipe.Allows portable and flexible operation without additional cameras.

    - Power Supply : Provides regulated voltage to ESP32, sensors, and relay modules.

###### Ensures stable system operation and prevents hardware damage due to voltage fluctuations.

    - Breadboard, Wires, and Connectors: Used for prototyping and connecting hardware components. Enables easy reconfiguration and debugging during development.

    - LED Indicators : Provide visual feedback for device status, relay switching, and gas alerts.Enhance user understanding and interaction with the system.

Roomate AI

#### Software Requirements

The software for RoomMate AI provides the intelligence, automation, and user interface of the system. It integrates gesture recognition, IoT control, web-based management, and real-time monitoring. Each software component is carefully chosen to ensure efficient performance, scalability, and user-friendly operation.

    - Python : Used as the primary programming language for gesture recognition. Handles image processing, gesture detection, and real-time analysis from the smartphone camera.Works in conjunction with OpenCV and MediaPipe libraries to process visual input and translate gestures into control signals.

    - OpenCV (Open Source Computer Vision Library) : Provides tools for real-time image and video processing.Detects hand movements and tracks gestures with high accuracy.Supports integration with Python for smooth execution of computer vision tasks.

    - MediaPipe : Framework designed for real-time gesture and pose recognition. Converts camera input into actionable data for controlling appliances.

Works alongside OpenCV to map hand gestures to specific commands (e.g. turning Appliances on/off)

    - Arduino IDE : Used to program the ESP32 microcontroller. Allows uploading firmware to ESP32 to control relay modules, read sensor data, and communicate with the web app.Supports multiple programming languages (C/C++) for microcontroller operations.

    - Web Application (HTML, CSS, JavaScript) : Provides a user interface for manual control of appliances.Displays real-time device status, sensor readings, and alerts.Enables users to control lights, fans, and monitor gas levels remotely via a browser.

    - Backend Server (Flask / Node.js) : Handles communication between the ESP32 hardware and the web application.Manages real-time updates, device commands, and sensor data transmission.Ensures smooth interaction between hardware inputs and software interface.

    - Browser / Mobile Device : Acts as the interface for the web application and gesture control.

Captures video input for gesture recognition. Enables remote monitoring and control from anywhere.

Roomate AI

#### Functional Requirements (Detailed)

The functional requirements of RoomMate AI define the capabilities and behavior of the system, ensuring that it meets the objectives of smart room automation, safety, and user convenience. These requirements describe how hardware and software components interact to perform specific tasks.

    - Gesture-Controlled Appliance Operation : Users can turn ON/OFF lights and fans using hand gestures captured through a smartphone camera. The system uses OpenCV and MediaPipe to recognize gestures in real-time. Gesture commands are mapped to specific appliance actions for intuitive and hands-free control.

    - IoT-Based Device Control : ESP32 microcontrollers, in combination with relay modules, enable wireless control of appliances.Provides reliable communication between the microcontroller and the web application, ensuring devices respond accurately to user commands.

Supports simultaneous control of multiple appliances, maintaining real-time responsiveness.

    - Gas Leakage Detection and Alerts : The system continuously monitors the room using a gas sensor (e.g., MQ-2 or MQ-5).When gas levels exceed a safe threshold, instant alerts are sent to the web application and optionally via notifications.Ensures user safety by providing timely warning of hazardous conditions.

    - Web Application Functionality : Users can manually control appliances using the web dashboard.Displays real-time device status, sensor readings, and safety alerts.

###### Supports remote monitoring and control, allowing users to interact with their room from any location.

    - Real-Time Monitoring and Feedback : The system provides continuous monitoring of all connected devices and sensors.Users can view logs of appliance usage, gas sensor readings, and alert history on the web interface.Provides visual and optional audio indicators for status updates and safety warnings.

    - Automation and Behavior Adaptation : Observes patterns of user activity, such as routine switching of lights or fans.Automatically performs repetitive tasks based on learned habits, such as turning off idle appliances or adjusting fan speeds.Helps reduce energy consumption and enhance user convenience.

Roomate AI

### Chapter 3 Diagram

      - 3.1 Circuit Diagram

      - 3.2 System Design

Roomate AI

  - Circuit Diagram

FIG: 1

Roomate AI

Circuit Diagram Description

The RoomMate AI circuit is centered on the ESP32 microcontroller, which acts as the main controller. All sensors and actuators are connected to its GPIO pins, while a 5 V DC power supply provides power to the ESP32, sensors, and relay module. All grounds are kept common for proper operation.

Sensors

- PIR Motion Sensor: Connected to a digital pin of the ESP32, it detects human movement and is used to control lights or trigger logs.

- MQ-2 Gas Sensor: Connected via an ADC pin, it monitors gas levels and sends alerts if unsafe concentrations are detected.

Actuators

- Relay Module: Connected to a digital output pin, it controls high-power devices like a light bulb or fan by switching them ON/OFF safely.

- DC Motor (Fan): Connected through the relay output, providing simple ON/OFF ventilation control.

- Buzzer: Connected to a GPIO pin, used to produce sound alerts during events like gas leakage.

Operation

- Sensors continuously feed data to the ESP32.

- The ESP32 processes inputs and, based on conditions, triggers outputs (relay or buzzer).

- For example, motion detected by the PIR sensor turns ON the light, while gas detection by the MQ-2 activates the buzzer and fan.

Safety Notes

- The relay isolates low-voltage ESP32 signals from high-voltage appliances.

- Proper wiring is essential, and high-voltage connections should never run on a breadboard.

Summary

- This circuit integrates sensing, processing, and actuation to enable smart room automation. It allows RoomMate AI to detect motion, ensure safety with gas monitoring, and control appliances efficiently.

Roomate AI

2

1

7

3

4

5

6

8

System Design

1. ESP32 Board → Main controller for automation & IoT connectivity

2. Relay Module → Controls appliances (Fan, LED light)

3. Li-ion Battery (18650) → Portable power source

4. Mini Fan → Simulates appliance control

5. LED Strip → Smart lighting demonstration

6. Gas Sensor (MQ) → Detects harmful gases/smoke

7. PIR Motion Sensor → Detects human presence

8. Buzzer → Gives alarm on danger (gas/motion events)

9. Breadboard Wiring → For easy prototyping & connections

Roomate AI

## Chapter 4 : Data Dictionary

    - 4.1 Data Source

    - 4.2 Data types

    - 4.3 Data & Organization

Roomate AI

### Chapter 4 : Data Dictionary

#### Data Source

RoomMate AI gathers data from multiple sources to make intelligent decisions about room automation, device control, and user interaction. The data is collected in real-time and stored systematically to ensure reliability, traceability, and historical analysis. The main data sources include IoT sensors, user inputs, camera and gesture inputs, system logs, and configuration files. Each of these sources plays a vital role in the functionality of the system.

- IoT Sensor Data : RoomMate AI is equipped with various sensors to monitor the environment and detect the status of the room. These sensors constantly provide raw data, which is then processed to trigger automation rules or alerts.

  - PIR Motion Sensor (HC-SR501): This sensor detects motion in the room. When motion is detected, it sends a binary signal to the ESP32 controller, which then decides whether to turn on lights or trigger alarms based on predefined rules. The motion data is timestamped and stored chronologically in a CSV file for future analysis. For example, a motion detected might be represented as 1, while no motion is 0.

  - MQ-2 Gas Sensor: This sensor monitors harmful gases like carbon monoxide (CO), LPG, and smoke. The sensor outputs gas concentration in parts per million (ppm), which RoomMate AI reads continuously. If the gas level exceeds a threshold defined in the configuration file, an alert is triggered and recorded in the logs. For instance, a reading might be GasLevel = 250 ppm, which would trigger an alert if the safe limit is 200 ppm.

  - Temperature and Humidity Sensor (DHT11/DHT22): These sensors monitor environmental conditions in the room. The data collected includes temperature in degrees Celsius and humidity in percentage. The ESP32 reads these values periodically, and if the readings exceed user-defined thresholds, automation rules are applied. For example, if the temperature exceeds 28°C, the system may automatically turn on the fan. All readings are stored in separate CSV files (Temperature.csv and Humidity.csv) for logging and historical analysis.

  - Relay Module Status: RoomMate AI monitors the status of connected appliances such as lights and fans through relay modules. The relay module provides a digital ON/OFF status to the ESP32, which logs the device state and performs actions based on automation rules.

Roomate AI

#### 2. User Input Data

RoomMate AI allows users to interact with the system through multiple interfaces, primarily the mobile application and optionally voice commands.

  - Mobile Application: The mobile app enables users to control appliances remotely, set schedules, and adjust device preferences. When a user sets a schedule, such as turning on the fan at 6 PM, the app sends this instruction to RoomMate AI, which processes it according to predefined rules. These commands are recorded in configuration files (user_settings.json) and logged for reference.

  - Voice Commands: For hands-free operation, RoomMate AI can accept voice commands through a microphone. Spoken commands are converted into text using a speech recognition module, and the system interprets the instruction. For instance, a user saying “Turn off the light” will result in the system turning off the light and logging the action in the VoiceCommands.log. Voice commands are also validated against allowed actions to prevent errors or conflicts.

These user input sources enhance the flexibility and usability of RoomMate AI, allowing users to interact naturally and remotely.

##### Camera and Gesture Input

RoomMate AI incorporates hand gesture recognition for intuitive control of devices. A camera captures images or video frames of the user’s hand movements, which are then processed using OpenCV and Mediapipe to detect gestures. Each gesture corresponds to a specific action, such as turning on the light, increasing fan speed, or adjusting brightness.

  - Gesture data is stored in the GestureImages folder for future reference and AI training.

  - Each captured image is timestamped and linked to the resulting action in the DeviceUsage.log.

  - This allows RoomMate AI to continuously improve gesture recognition accuracy and maintain a record

###### of user interactions.

For example, a swipe-up gesture may be interpreted as “increase light brightness,” while a swipe-down gesture could mean “decrease brightness.”

Roomate AI

#### Data Types

RoomMate AI handles different types of data to monitor the room, control devices, and maintain records. The main data types are:

##### Numerical Data

Collected from sensors like temperature, humidity, gas, and motion detectors. These readings are used to make decisions, such as turning on the fan if the temperature is too high. Example: Temperature = 27°C, GasLevel = 250 ppm.

##### Binary Data

Represents ON/OFF or yes/no states of devices and sensors. Example: Fan = ON, Light = OFF, MotionDetected = 1. This helps RoomMate AI know the current status of appliances.

- Textual Data

###### Includes logs, error messages, and notifications. Example: "Fan turned ON at 15:02". Textual data helps

in monitoring system activity and troubleshooting errors.

##### Multimedia Data

Images captured from the camera for gesture recognition and room monitoring. Example: hand gestures to control light brightness. Stored as .jpg or .png files.

- Structured Data

###### Organized data stored in CSV or JSON format. This includes sensor readings, device usage, and user

settings.

Roomate AI

#### Data Storage & Organization –

RoomMate AI generates and collects a large volume of data from its sensors, user inputs, camera, and system operations. Proper storage and organization of this data is essential for smooth system operation, historical tracking, troubleshooting, and analysis. This section explains the structure, formats, and storage mechanisms for all data in the system.

#### Folder and Directory Structure

All data collected and generated by RoomMate AI is organized into a hierarchical folder structure, making it easy to locate, manage, and back up. The main directory, named /RoomMateAI_Data/, contains subfolders for sensors, logs, multimedia, configuration, and backups.

  - Sensors Folder: This folder stores raw data collected from IoT sensors, including temperature, humidity, gas levels, and motion detection. Each sensor has its own CSV file where data is recorded with a timestamp for every reading. For example, Temperature.csv contains entries like 2025-09-26 10:00: 27°C. The folder structure allows chronological tracking and easy retrieval for analysis.

  - Logs Folder: All operational and system logs are stored here. This includes device usage logs, automation history logs, and error logs. Each log entry contains the timestamp, type of event, and details of the action. For example, "Fan turned ON at 15:02 due to temperature threshold" is recorded in DeviceUsage.log. The logs folder is critical for monitoring system performance and debugging.

  - Multimedia Folder: This folder stores images and videos captured by the camera for gesture recognition and AI training. Hand gesture images are saved in the GestureImages/ subfolder, while snapshots of the room environment are stored in CameraSnapshots/. Each file is timestamped to maintain chronological order and link it with the corresponding action logged.

  - Config Folder: User preferences and sensor thresholds are maintained in JSON configuration files within this folder. user_settings.json contains schedules and appliance preferences, while sensor_thresholds.json stores the threshold values for temperature, gas, or motion that trigger automation rules. These files are encrypted to ensure security and privacy.

Roomate AI

#### File Naming Conventions

To maintain order and simplify data retrieval, RoomMate AI follows consistent file naming conventions:

  - Timestamped Files: Sensor data files include timestamps in the filenames for easier chronological tracking. Example: Temperature_20250926_1000.csv

###### Daily Logs: Logs are maintained on a daily basis, e.g., DeviceUsage_20250926.log, which helps

separate events and simplifies archiving.

  - Gesture Images: Stored using descriptive names and timestamps, e.g., SwipeUp_20250926_1015.jpg.

  - Backup Files: Daily backups are named according to the date, e.g., backup_20250926.zip.

This structured approach prevents file conflicts, ensures easy data navigation, and facilitates automated

processing by the AI controller.

#### Data Formats

  - CSV Files: Used for structured sensor data like temperature, humidity, gas, and motion. CSV allows easy import into analysis tools and quick reading by the AI controller.

  - JSON Files: Configuration and user preferences are stored in JSON format. This allows hierarchical

###### organization of data (like schedules or threshold values) and easy updates.

  - Log Files (TXT/LOG): Human-readable text files store chronological logs of device usage, automation history, and system errors.

  - Multimedia Files (JPG/PNG): Images captured for gesture recognition or environmental monitoring are stored in standard formats suitable for AI processing.

###### ZIP Files: Backups are compressed to save storage space while maintaining the directory hierarchy for

recovery.

#### Data Access and Management

  - Read/Write Access: Sensor data is continuously written to CSV files, while logs are appended in real- time. The AI controller reads these files as needed to make automation decisions.

  - Configuration Access: JSON files are read at system startup and dynamically updated when user

preferences change. Encryption ensures unauthorized access is prevented.

  - Multimedia Management: Images for gesture recognition are read by the AI module for processing,

and old images are archived to optimize storage.

Roomate AI

### Chapter – 5 Limitations & Future

Enhancement

Roomate AI

#### Chapter 5 Limitation & Future Enhancement Limitations

##### Hardware Dependence:

RoomMate AI relies heavily on sensors, cameras, and relay modules to gather environmental data and control devices. Any hardware malfunction, such as a faulty gas sensor or a camera with poor resolution, can affect the system’s accuracy, responsiveness, and overall reliability. Regular maintenance is required to ensure consistent performance.

##### Limited Gesture Recognition:

The hand gesture detection module depends on camera clarity, lighting conditions, and predefined gestures. Complex gestures or subtle hand movements may not be recognized correctly, leading to incorrect device control.

- Connectivity Issues:

###### The mobile app and AI modules require a stable WiFi connection. Network interruptions or latency

can delay commands, prevent remote access, or cause synchronization issues between devices.

- Scalability Constraints:

Currently, RoomMate AI is optimized for a single-room setup. Expanding the system to multiple rooms or large spaces requires additional sensors, relays, and possibly multiple ESP32 controllers, which can increase complexity and cost.

- Limited AI Learning Capability:

The behavior-learning module is rule-based. While it can automate simple routines, it cannot dynamically learn complex or irregular user habits without manual configuration. This limits personalization and adaptive intelligence.

- Data Storage Limitations:

Continuous logging of sensor data, multimedia from gesture recognition, and system logs can consume significant memory over time. Without regular backups, archival, or cloud integration, storage limitations can affect long-term data retention and historical analysis.

- Environmental Limitations:

Extreme environmental conditions, such as poor lighting, smoke, or dust, can affect sensor accuracy and gesture recognition. The system may not function optimally under such conditions.

Roomate AI

#### Future Enhancements

Advanced AI Integration:

Implement machine learning algorithms to predict user behavior and preferences. The system could learn when a user typically adjusts the lights or fan and automate actions without manual scheduling.

Multi-Room Expansion:

###### Extend the system to manage multiple rooms or an entire home. This would involve integrating centralized

control dashboards, multi-node communication, and coordinated automation between rooms.

Enhanced Gesture Recognition:

Improve the gesture recognition module to detect more complex gestures and operate accurately under varying lighting conditions. Integration of infrared or depth cameras could further improve recognition reliability.

Voice Command Improvements:

Incorporate advanced natural language processing (NLP) for multi-language support, context-aware responses, and more natural conversational interactions with the AI assistant.

Cloud Storage & Analytics:

###### Integrate cloud storage to store sensor readings, logs, and multimedia data. Cloud-based analytics can

provide historical trends, energy consumption reports, and predictive maintenance alerts.

Energy Efficiency Features:

Implement intelligent energy-saving suggestions. For example, the system could turn off lights or fans automatically when a room is unoccupied or optimize device usage based on environmental conditions and user habits.

Integration with IoT Ecosystem:

Enable interoperability with other smart home devices and platforms, such as smart thermostats, security

###### cameras, or voice assistants, creating a fully connected and automated home ecosystem.

Robust Security Features:

Enhance data and network security with encryption, secure authentication, and regular updates to prevent unauthorized access and ensure user privacy.

Mobile App Enhancements:

###### Improve the mobile app interface for easier device management, real-time alerts, and remote control

Roomate AI

###### Conclusion

Roomate AI

#### Conclusion

RoomMate AI is a comprehensive smart room assistant designed to combine IoT technology, artificial intelligence, and user-friendly interfaces to create an intelligent, adaptive, and automated living environment. The system integrates multiple hardware components, including sensors (temperature, humidity, motion, gas), relay modules, and cameras, along with software modules for gesture recognition, voice commands, and automation logic. Together, these components enable real-time monitoring, intelligent decision-making, and seamless control of devices such as lights, fans, and alarms.

The project demonstrates the ability of RoomMate AI to respond intelligently to environmental changes and user preferences. Through the collection of numerical, binary, textual, and multimedia data, the system is capable of analyzing patterns, triggering automated responses, and maintaining detailed logs for system monitoring and future analysis. The integration of behavior-learning modules allows the system to follow pre-defined routines and adjust device operation based on observed user habits, thereby improving comfort and efficiency.

One of the key achievements of the project is the successful implementation of gesture-based control and mobile app integration, which offers a natural and intuitive interface for users. This ensures that users can control their environment even without direct interaction with devices, increasing accessibility and convenience. Additionally, the systematic organization of data in CSV, JSON, and multimedia formats, along with secure backups, ensures that RoomMate AI maintains high reliability, data integrity, and traceability.

Despite its successes, the project has certain limitations. Hardware dependence, limited scalability, connectivity constraints, and rule-based AI learning restrict the system’s full potential. Gesture recognition accuracy can vary under different lighting conditions, and continuous data logging may require careful management to prevent storage overflow. Recognizing these limitations has provided valuable insights into how future improvements can enhance the system’s intelligence, reliability, and adaptability.

Looking forward, the system can be enhanced by integrating advanced machine learning algorithms, expanding to multi-room or full-home automation, implementing cloud-based storage and analytics, improving gesture and voice recognition, and enabling energy-efficient operation. These enhancements will allow RoomMate AI to learn and adapt dynamically to user behavior, optimize energy consumption, provide predictive insights, and integrate with other smart home devices for a fully connected and intelligent environment.

In conclusion, RoomMate AI successfully demonstrates the practical application of IoT, AI, and automation technologies to create a smart, adaptive, and efficient room environment. The project highlights the potential for future development in smart home systems, showcasing how technology can enhance daily life by improving comfort, safety, convenience, and energy efficiency. RoomMate AI serves as a strong foundation for further research and development in the field of intelligent home automation.